%---------------------------- HIPh-template.tex -------------------------%|

\documentclass{article}

\usepackage{hiph-art}

%\usepackage{graphicx}

%\usepackage{amssymb}

% -----------------------------------------------------------------------%|

% Template TeX file for                                                  %|

%                                                                        %|

%             Acta Physica Hungarica A                                   %|

%                                                                        %|

%                 Heavy Ion Physics                                      %|

%                                                                        %|

% PLEASE, KEEP THIS HEADER WHEN COMPOSING YOUR OWN TeX SOURCE.           %|

%                                                                        %|

% ---------------------------------------------------------------------- %|

%                                                                        %|

% Please, submit the following data with the article:                    %|

% ===================================================                    %|

% Communicating author:                                                  %|

% His/her e-mail:                                                        %|

% Date of submission:                                                    %|

%                                                                        %|

% (Also, please don't forget about the ``keywords'' and                  %|

%  ``PACS numbers'' below.)                                              %|

% ---------------------------------------------------------------------- %|

%                                                                        %|

\volnumber{19} \issuenumber{1} \edyear{2004}                             %|

\frompage{000} \topage{000}                                              %|

\recrevdate{1 January 2004}                                              %|

%________________________________________________________________________%|

\def\rightmark{Applications of the gamma-ray detection technique}

\def\leftmark{G.~Curutchet et al.}

\title{Applications of the gamma-ray detection technique in environmental sciences}

\authors{

{G.~Curutchet$^{1,2}$, D.E.~Di~Gregorio$^{1,2,3}$,

J.O.~Fern\'andez~Niello$^{1,2,3,a}$, R. Gargarello$^{1,2}$, H.

Huck$^{1,3}$ and H. Somacal$^{1}$

%

%

\index{Curutchet, G.} % Abbreviated names of the author(s),

\index{Di~Gregorio, D.E.} % to be inserted for use in the volume index

\index{Fern\'andez~Niello, J.O.}

\index{Gargarello, R.}

\index{Huck, H.}

\index{Somacal, H.}

}\\[2.812mm]

{\normalsize \hspace*{-8pt}$^1$ Escuela de Ciencia y

Tecnolog\'{\i}a, Universidad Nacional de General San Mart\'{\i}n,

Mart\'{\i}n de Irigoyen 3100, 1650 San Mart\'{\i}n, Buenos Aires, Argentina\\[0.2ex]

\hspace*{-8pt}$^2$ Consejo Nacional de Investigaciones Cient\'{\i}ficas y T\'ecnicas, Argentina\\

\hspace*{-8pt}$^3$ Laboratorio TANDAR, Comisi\'on Nacional de Energ\'\i a At\'omica,\\

Av. General Paz 1499, 1650 San Mart\'{\i}n, Buenos Aires,Argentina\\

%

}}

\abstract{Gamma-detection systems represent a powerful tool to measure

radioisotopes commonly present in environmental samples.  Our

group has recently undertaken a research program that takes

advantage of an ultra-low gamma detector in applications related

to the environmental impact in aqueous systems. Two different

applications of this technique are being carried out in our

laboratory:

1) Measurements of $^{210}$Pb and $^{137}$Cs concentrations in sediment samples

from Santiago River (Buenos Aires, Argentina).

2) Development of a possible way for the bioremediation of uranium contaminated effluents.}

\keyword{$^{210}$Pb, $^{137}$Cs , sedimentation rate,

bioremediation, $^{235}$U, $^{238}$U}

\PACS{92.40.Gc}

\makeindex

\begin{document}

\maketitle

Gamma-ray spectroscopy has been used for a wide range of

environmental applications including sediment transport

investigations and studies on contaminations arising from the

nuclear industry. The basis of this technique is the measurement

of gamma-ray emissions from primordial or anthropogenic

radionuclides. Indeed, $^{210}$Pb and $^{137}$Cs activities (with

half-lives of 22.3 and  30.2 years, respectively), are usually

applied to studies of sediment deposition in aquatic environments.

Moreover, provided a low-background facility is available,  this

technique can be also used to trace tiny amount of uranium present

in a solution. Therefore, we started a research program towards

the identification of use of bacteria or fungal species in the

biosorption of uranium from aqueous solutions.

In the different series of measurements, that were performed at

the Tandar Laboratory in Buenos Aires, we used two high-purity

germanium detectors. One of these has a planar configuration for

the low-energy gamma-ray spectrum, and the other was a

40\%-efficient high-purity germanium detector built by Princeton

Gamma-Tech for energies ranged from 100 to 2200 keV. The detectors

were surrounded by approximately 50~cm of lead bricks to provide

shielding against radioactive background. An inner shielding

provided by a 2.5-cm thick Marinelli beaker made of archaeological

Roman lead ($\sim$2000 years-old) was used to improve the

sensitivity of the planar detector. This shielding allowed us to

reach detection limits as low as 0.1~Bq/kg.

In the first application, sediment cores were collected at two

sites along an area located between latitude 34$^{\circ}$~50' and

34$^{\circ}$~53' S and longitude between 57$^{\circ}$~54' and

57$^{\circ}$~57' W (sites A and B, respectively), where the

Santiago River joins the La Plata River in Argentina. The lengths

of the cores collected for this study vary from 170 to 240 cm.

Sliced layers of sediments (taken at 5 cm intervals down to the

bottom) were homogenized, dried, and then reduced to a fine powder

before the analysis. Sediment samples of about 10 g were held in

plastic vials for  gamma-ray counting.

The variation of the concentration of $^{210}$Pb with the  depth

for both sites  allows one to obtain sufficient information to

determine time scales for the sediments. We found that the

measured $^{210}$Pb concentrations vary exponentially with the

depth suggesting that the sediments remained unmixed and linear

sedimentation rates of (0.6$\pm$0.1)~cm/year and

(0.9$\pm$0.1)~cm/year were determined for the site A and B,

respectively. On the other hand, the $^{137}$Cs results were not

useful for confirming dates and fully assessing sediment

accumulation.

The second of the mentioned applications deals with bioremediation

of effluents contaminated with uranium. We developed two

approaches: Firstly, we studied the adaptation of the bacteria

Acidithiobacillus thiooxidans to high uranium concentrations and

its potential use as biocatalyst for the aerobic indirect

reduction/precipitation of this element. In this case, the

quantitative determination of the uranium present in liquid and

solid samples is done by means of the gamma-ray identification of

the products stemming from the decay of $^{235,238}$U. This

research is still in progress.

Secondly, acid and uranium resistant fungal species (Acremonium

and Aphanocladium) biomass were used for biosorption of uranium

from aqueous solutions. The quantitative kinetics of the uptake

process (using living or desiccated cells) was measured by means

of the $^{238}$U and $^{235}$U decay peaks at 92.2~keV and

185.7~keV, respectively. The gamma-ray determination was

correlated with the standard arsenazo(III) method. A very good

correlation was found, but the gamma-ray has some advantages. For

instance, simple and direct measurements are possible in both,

liquid and solid (biomass) samples. Preliminary results seem to

indicate that the Aphanocladium sp. presents a higher remotion

capability than the Acremonium sp. for biomass samples.

\begin{notes}

\item[a]

Permanent address: Laboratorio TANDAR, CNEA, Argentina;\\

E-mail: fniello@tandar.cnea.gov.ar

\end{notes}

\vfill\eject

\end{document}